HPC - Use of Cobra “walk behind” brush cutter. Recommendations and safe practice notes. Please read in conjunction with WBCS Risk Assessment and Site Safety Sheets Based on LANTRA booklet – 04/05/07 Use of harness mounted brush cutters WBCS original Brush Cutter RA October 2013 Updated August 2015 and Nov 2018 Further edits added in January 2023 covering use of the Cobra machine.

| 1. Preparing for brush cutter operations |  |
| --- | --- |
| - Risk Assessment - Other workers on site - Public access to site - Lookouts and lone working - Communication - First Aid - Location for emergency services | - Ensure that a risk assessment has been carried out and the significant findings recorded. (See WBCS Risk Assessments) - All workers on site to be made aware of and comply with the controls identified. Highlight during start of work briefing. - Erect warning signs and barriers as necessary - especially if the public are involved. - The use of lookouts (spotters) is recommended. Operators should not normally work alone. - Agree a method of communication between the operator and the look out before starting work. Usually hand signals - A First aider is needed on site, No first aider = no work A First Aid kit with eye wash should be available on site. - In case of emergency, be able to provide the emergency services with adequate information about your location. (For Furze Hill and Village Hall – RG18 9WL) |
| 2. Machine checks by operator before and during use. |  |
| - Safety features - Controls working correctly. - General condition check - Fuel and fuelling - Fire blanket - Starting the machine. - Report any problems | - Check all safety features are present and adjusted. Blade guard and engine covers in place. - Check that all the controls are working correctly, On/off switch, throttle lock, choke, and recoil starter - Check that there are no loose or damaged parts. Check condition of cutter strings. Replace as necessary. - Fuel tank filled with correct fuel. (Four stoke only) Top up engine oil as necessary – use dip stick. - Fire blanket to be available near fueling point. - Machine should be started well clear of the refuelling point. - Report any problems or damage to the machine. . |

| 3. Operating the brush cutter. |  |  |
| --- | --- | --- |
| - Agree the area to be cut. - Wear suitable PPE - Check the site. - Avoid contact with damaging materials. - Check for toxic plants. - Check if cutter rotates on tick over. - Ensure safe distances maintained. - Time limits for safe working | - Agree with the site leader the area that is to be cut. - Wear suitable PPE –Helmet with mesh face shield, eye and ear protection, gloves, leg protection and boots. - Check the Site conditions -be aware of the unexpected! - Avoid cutting, or contact with, materials that may damage cutting head – e.g. metal or masonry – glass, syringes, excrement etc., tree stumps etc. - Check site for, and avoid, rare or toxic plant species. - Do not use the cutter if the head rotates on “tick-over” or if there is excessive vibration from the cutting head. - Ensure a safe working distance (at least15 m radius / 50ft) between brush cutter and other workers. - Agree break times – either per tank of fuel or timed sessions for swop between operators |  |
| 4. Routine maintenance carried out during and after each work session |  |  |
| - Check safety features present - Check all controls working correctly. - Check for loose and damaged parts. - Engine starts and runs evenly. - Check cutter strings. - Check air filter clean and or replace. - Check and adjust if necessary. - Check and top up engine oil. - Brush down any loose material after each work session |  | - Blade guard, engine cover and H&S decals in place - On/off switch, throttle, throttle lock, choke and recoil starter - Take note of cutter mounting - check hex screw. - Cutters should not turn when in “tick over” – adjust as needed. - Check length of strings – replace as necessary. - Air filter, fuel tank filter and fuel line. - check for blockages - Spark plug and carburettor - Oil change – see engine manual for oil change periods. - Specially material lodged near air vents. |
| Other information |  |  |
| Legislation involved - HASAW 1974, PUWER 1998, Control of Noise at Work & Control of Vibration at work Regs both 2005 |  | HSE/AFAG leaflets AFAG 802 Emergency planning AFAG 203 Clearing saw. – a separate qualification is recommended for this tool. All at :- http://www.hse.gov.uk/treework/index.htm |